Keep this sheet with you during all map reading and navigation activities.

| OS MAP SYMBOLS. Feature; How it appears. Public footpath; Green dashed line. Public bridleway; Long green dashes. Nat. Park boundary; Orange dot-dash line. A-road (dual); Red double line. B-road; Orange/brown line. Track (unsurfaced); Dashed double line. River / stream; Blue wavy line. Lake / reservoir; Filled blue area. Marsh / wetland; Blue tick-mark pattern. Coniferous woodland; Dark green triangles. Non-conif. woodland; Light green circles. Contour line; Brown line + height number. Spot height; Black dot with number. Trig point; Blue triangle with dot. Car park (P); Blue square, white P. Youth hostel (YH); Blue rect — YH. Campsite; Blue open triangle. Church with tower; Black cross + square. Public house (PH); PH in black text. Info centre (i); Blue square — i.. |  | SCALE AND DISTANCE. Formula: Map measurement x scale = real distance Scale; 1 cm =; 4 cm =. 1:25,000; 250 m; 1 km. 1:50,000; 500 m; 2 km. Example: 8 cm on 1:25,000 = 8 x 250 m = 2,000 m = 2 km. GRID REFERENCES. Read EASTINGS first (along), then NORTHINGS (up) Mnemonic: Along the corridor, then up the stairs Type; How to give it. 4-figure (Pass); Easting (34) + Northing (55) = 3455. Identifies a 1 km square.. 6-figure (M/D); Estimate tenths across (2)=342, tenths up (4)=554. Result: 342554. Locates 100 m area... CONTOUR LINES. Interval on OS Explorer: 10 metres per contour line Close together = steep slope Far apart = gentle slope Ring of contours = hill top or summit V pointing uphill = valley or stream V pointing downhill = spur or ridge. |
| --- | --- | --- |

| NAISMITH'S RULE. Flat time: Distance / 5 = hours Ascent time: Height / 600 = hours (10 min per 100 m) Total = Flat time + Ascent time Distance; Flat time; Ascent; Add. 5 km; 1h 00min; 100 m; + 10 min. 7.5 km; 1h 30min; 200 m; + 20 min. 10 km; 2h 00min; 300 m; + 30 min. 12.5 km; 2h 30min; 400 m; + 40 min. 15 km; 3h 00min; 500 m; + 50 min. 20 km; 4h 00min; 600 m; + 60 min. Limitations: Add time for: heavy pack, boggy ground, rest stops, mixed-ability group, bad weather.. THE STOP METHOD — IF LOST. S — STOP: Do not keep walking. Keep the group together and calm. T — THINK: When did you last know exactly where you were? O — OBSERVE: Look for features around you that match the map. P — PLAN: Identify your position, then agree a plan before moving.. THE COUNTRYSIDE CODE. Leave no trace — take all litter home Close all gates behind you Keep dogs on leads near livestock Stay on marked footpaths Respect wildlife, farmland and other users. |  | NAVIGATION TECHNIQUES. 1. Orient the map: Rotate until map features align with ground. Use compass to set north. 2. Track progress: Tick off landmarks as you pass. Use Naismith to estimate distance covered. 3. Attack points: Navigate to a clear feature near your target, then take a precise bearing. 4. Handrail features: Follow a linear feature (wall, stream, track) as a guide to direction. 5. Pacing: Count double-steps. Approx 65 paces = 100 m on flat ground. 6. Tick features: Identify features along your route and tick off as you pass to confirm position.. ROUTE CARD CHECKLIST. Leg number Start six-figure grid reference End six-figure grid reference Distance (km) Height gain (m) Naismith time (minutes) Terrain and navigation notes Emergency contact Escape route (Distinction). GRADE REMINDER. Grade; What you need. PASS; 4-fig refs, basic plan, navigate with help. MERIT; 6-fig refs, Naismith timings, navigate on time. DISTN; Justified plan + escape routes, independent nav.. |
| --- | --- | --- |